32F Frequency Converter Instruction

wiring diagram：

bare size：long135mm*wide85mm*high86mm

size：long139mm*wide86mm*high90mm

Parameter specification：

| parameter | Parameter specification | Parameter rang |  | default | For note |
| --- | --- | --- | --- | --- | --- |
| F0 | Working frequency | 0～400Hz |  | 50Hz |  |
| F1 | Working frequency source | 0，Panel keyboard 1, external potentiometer 2, panel potentiometer |  | 2 |  |
| F2 | Working frequency source | 0，Panel keyboard 1，external port |  | 0 |  |
| F3 | Stop way | 0，slow stop 1，braking stop 2，Inertia stop |  | 0 |  |
| F4 | Maximum operating frequency | 0～400Hz |  | 65Hz |  |
| F5 | Lowest operating frequency | 0～400Hz |  | 5Hz |  |
| F6 | Increase speed of Frequency | 1～500Hz/S |  | 20Hz/S |  |
| F7 | Decline speed of Frequency | 1～500Hz/S |  | 20Hz/S |  |
| F8 | Braking time | 0-3S |  | 0.3S |  |
| F9 | Braking coefficient | 0～60％ |  | 20％ |  |
| F10 | Lowest output voltage | 0～80 V |  | 10V |  |
| F11 | Middle voltage | 0～220 V |  | 65 V |  |
| F12 | highest output voltage | 0～220 V |  | 220V |  |
| F13 | Lowest frequency | 0～400Hz |  | 3Hz |  |
| F14 | Middle frequency | 0～400Hz |  | 15Hz |  |
| F15 | Highest frequency | 0～400Hz |  | 50Hz |  |
| F16 |  | 0 | Mi1：Forward/Stop; Mi2: Reverse/Stop | 0 |  |
|  | Mi1 and Mi2 function set | 1 | Mi1：Run/Stop; Mi2: Reverse/ Forward |  |  |
|  |  | 2 | Mi1,Mi2,Mi3:three-wire system |  |  |
|  |  | 3 | Mi1：Forward/Stop |  |  |
| F17 | Mi1 Function Set | 0: non-function 1：stop when off 2：stop when on 3：Abnormal clear |  | 0 |  |
| F18 | Mi2 Fun Set | 0：non 1：Stop when off 2：Stop when on 3：Abnormal clear 4：Speed BIT0 5：Speed BIT1 6：Speed BIT2 7：Speed BIT3 8：forward move 9：reversal move |  | 0 0 | Mi5 with MO Function alternative, can’t use meanwhile |
| F19 | Mi3 Fun Set |  |  |  |  |
| F20 | Mi4 Fun Set |  |  |  |  |
| F21 | Mi5 Fun Set |  |  |  |  |
| F22 | Speed 1 | F5～F4 |  | 0 |  |
| F23 | Speed 2 | F5～F4 |  | 0 |  |
| F24 | Speed 3 | F5～F4 |  | 0 |  |
| F25 | Speed 4 | F5～F4 |  | 0 |  |
| F26 | Speed 5 | F5～F4 |  | 0 |  |
| F27 | Speed 6 | F5～F4 |  | 0 |  |
| F28 | Speed 7 | F5～F4 |  | 0 |  |
| F29 | Speed 8 | F5～F4 |  | 0 |  |
| F30 | Speed 9 | F5～F4 |  | 0 |  |
| F31 | Speed 10 | F5～F4 |  | 0 |  |
| F32 | Speed 11 | F5～F4 |  | 0 |  |
| F33 | Speed 12 | F5～F4 |  | 0 |  |
| F34 | Speed 13 | F5～F4 |  | 0 |  |
| F35 | Speed 14 | F5～F4 |  | 0 |  |
| F36 | Speed 15 | F5～F4 |  | 0 |  |
| F37 | Reach the frequency | F5～F4 |  | 0 |  |
| F38 | Mo function set | 0：Run H，Stop L 1: Equal set frequency H，else L 2: Frequency equal 0，H，else L 3：≧”F37” H，else L 4: Fault H，Normal L 5: AO |  | 0 | Mi5 with MO Function alternative, can’t use meanwhile |
| F39 | Current limit | 2～5A |  | 3.5A |  |
| F40 | Speed ratio | 1～120（1 is frequency） |  | 1 |  |
| F41 | Speed offset | 0～200 |  | 100 |  |
| F42 | Direction of timing | 0：Count down；1：Timing |  | 0 |  |
| F43 | Carrier frequency | 8～15kHz |  | 8 |  |
| F44 | power saving | 0～30 |  | 0 |  |
| F45 | ”F21” parameter 8 | F5～F4 |  | 0 |  |
| F46 | ”F21” parameter 9 | F5～F4 |  | 0 |  |

Press FUNC key，you see F 0，press ▲、▼key，Select F 0～F46，Press ENTER，show the Parameter value xx，use▲、▼to adjusting, press ENTER save，or press FUNC don’t save。

When F 1 is 0，motor running，▲key to Increase the frequency,▼key to decrease the frequency。

When F 1 is 1，motor frequency decided by impressed voltage or external potentiometer

Use F 6、F 7 when accelerate and decelerate，example：F 0＝50Hz，F 6＝10Hz，F 7＝25Hz，run after 5S，motor get 50Hz；when stop，after 2S，from 50Hz to 0。

Restore Factory Defaults ：turn off and press FUNC with ENTER key，turn on，Restore when show def.

fault code：

oL1，oL2: overdrive；               EC: over current Module to protect；

Et: over-temperature protection；      Lost: Communication is wrong.